V2V MULTI-SENSOR FUSION

COMMUNICATION SYSTEM

Complete Hardware Wiring Guide

| Document Type | Hardware Wiring Reference |
| --- | --- |
| Project | V2V ESP8266 Convoy System |
| Author | Deepak M |
| Version | 1.0 — Final |
| Date | 2025 |
| Classification | Portfolio — MIT Application |

github.com/deepakmurugan578-beep/V2V_-Communication-Multi-Sensor-Fusion-

# 1.  System Overview

This wiring guide provides complete, step-by-step hardware connection instructions for both vehicles in the V2V Multi-Sensor Fusion Convoy System. The system uses two ESP8266 NodeMCU v2 microcontrollers communicating via the ESP-NOW peer-to-peer protocol at approximately 1ms latency.

## 1.1  Power Architecture

Both vehicles use a Power Board V2 with regulated 3.3V, 5V, and 12V outputs. This eliminates the need for individual buck converters and ensures clean, stable voltage for all sensors.

| Voltage | Source | Powers | Notes |
| --- | --- | --- | --- |
| 3.3V | Power Board | MPU6050, OLED, IR Sensor, Buzzer | All logic sensors |
| 5V | Power Board | L298N logic VCC | Motor driver logic only |
| 12V | Power Board | L298N VIN (motor power) | Drive motors only |
| USB | Laptop/PC | ESP8266 NodeMCU only | Upload + Serial Monitor |
| GND | Common Rail | All components | Must share single GND rail |

✕ DANGER: NEVER connect USB and 12V battery simultaneously. USB provides power to ESP8266 during programming. Battery powers motors during operation. Connecting both can permanently damage the ESP8266.

⚠ WARNING: All components must share a COMMON GROUND. Connect Power Board GND, L298N GND, ESP8266 GND, all sensor GNDs, and battery negative to a single ground rail.

## 1.2  HC-SR04 Voltage Divider (Car 1 Only)

The HC-SR04 ECHO pin outputs 5V, but the ESP8266 D5 pin is 3.3V tolerant only. A voltage divider must be used to protect the ESP8266:

HC-SR04 ECHO ──[1kΩ]──┬──[1kΩ]── GND └── ESP8266 D5 (ECHO input)

This divides the 5V ECHO signal to approximately 2.5V — safely within the ESP8266 3.3V logic threshold. Use two 1kΩ resistors in series.

# 2.  Car 1 — Leader Wiring

Car 1 is the convoy leader, equipped with an HC-SR04 ultrasonic sensor for forward distance measurement, MPU-6050 for tilt and crash detection, OLED display, and a 4-wheel drive L298N motor system.

## 2.1  Complete Pin Map

| Pin | GPIO | Connected To | Purpose / Notes |
| --- | --- | --- | --- |
| D1 | GPIO5 | MPU6050 SCL + OLED SCL | I2C clock — shared bus |
| D2 | GPIO4 | MPU6050 SDA + OLED SDA | I2C data — shared bus |
| D3 | GPIO0 | L298N IN1 | Motor Left — Forward direction |
| D4 | GPIO2 | L298N IN2 | Motor Left — Backward direction |
| D5 | GPIO14 | HC-SR04 ECHO (via divider) | Ultrasonic echo — 3.3V safe via 1k+1k |
| D6 | GPIO12 | HC-SR04 TRIG | Ultrasonic trigger pulse |
| D7 | GPIO13 | L298N IN3 | Motor Right — Forward direction |
| D8 | GPIO15 | L298N IN4 | Motor Right — Backward direction |
| D0 | GPIO16 | L298N ENA | Left motor PWM speed control |
| RX | GPIO3 | L298N ENB | Right motor PWM speed control |
| TX | GPIO1 | FREE | Serial Monitor active on TX |
| A0 | ADC0 | NOT USED | Analog input — not connected |
| 3.3V | — | MPU6050 VCC + OLED VCC | From Power Board 3.3V rail |
| GND | — | All component GNDs | Common ground rail |
| USB | — | Laptop/PC | Power + programming + Serial Monitor |

⚠ WARNING: Disconnect the RX wire (connected to L298N ENB) before uploading code. The RX pin is used for serial programming. Reconnect after upload completes.

## 2.2  MPU-6050 Wiring (Car 1)

| MPU-6050 Pin | Connect To |
| --- | --- |
| VCC | Power Board 3.3V |
| GND | Common GND rail |
| SCL | D1 (GPIO5) |
| SDA | D2 (GPIO4) |
| XDA | Not connected |
| XCL | Not connected |
| ADO | GND (sets I2C address 0x68) |
| INT | Not connected |

ℹ NOTE: MPU-6050 and OLED share the same I2C bus (D1=SCL, D2=SDA). MPU-6050 uses address 0x68 and OLED uses address 0x3C — no conflict.

## 2.3  SSD1306 OLED Wiring (Car 1)

| OLED Pin | Connect To |
| --- | --- |
| VCC | Power Board 3.3V |
| GND | Common GND rail |
| SCL | D1 (GPIO5) — shared with MPU-6050 |
| SDA | D2 (GPIO4) — shared with MPU-6050 |

## 2.4  HC-SR04 Ultrasonic Sensor Wiring (Car 1)

| HC-SR04 Pin | Connect To |
| --- | --- |
| VCC | Power Board 3.3V |
| GND | Common GND rail |
| TRIG | D6 (GPIO12) — direct connection |
| ECHO | Junction of two 1kΩ resistors — see divider circuit above |

✕ DANGER: ECHO pin outputs 5V — connecting directly to ESP8266 will destroy the GPIO. Always use the 1kΩ + 1kΩ voltage divider between ECHO and D5.

## 2.5  L298N Motor Driver Wiring (Car 1 — 4WD)

| L298N Pin | Connect To |
| --- | --- |
| 12V / VIN | Power Board 12V — motor power |
| 5V / VCC | Power Board 5V — logic power |
| GND | Common GND rail |
| IN1 | D3 (GPIO0) — Left motor forward |
| IN2 | D4 (GPIO2) — Left motor backward |
| IN3 | D7 (GPIO13) — Right motor forward |
| IN4 | D8 (GPIO15) — Right motor backward |
| ENA | D0 (GPIO16) — Left motor PWM speed |
| ENB | RX (GPIO3) — Right motor PWM speed |
| OUT1/OUT2 | Left motor terminals (polarity sets direction) |
| OUT3/OUT4 | Right motor terminals (polarity sets direction) |

ℹ NOTE: PWM speed range is 0–255. Speed constants used: STOP=0, SLOW=80, MEDIUM=160, FAST=220. Adjust motor polarity connections (OUT1/OUT2 swap) if motor spins in wrong direction.

# 3.  Car 2 — Follower Wiring

Car 2 is the convoy follower, equipped with an IR obstacle sensor, MPU-6050 for tilt and crash detection, OLED display, an active buzzer for audio alerts, and a 2-wheel drive L298N motor system.

## 3.1  Complete Pin Map

| Pin | GPIO | Connected To | Purpose / Notes |
| --- | --- | --- | --- |
| D1 | GPIO5 | MPU6050 SCL + OLED SCL | I2C clock — shared bus |
| D2 | GPIO4 | MPU6050 SDA + OLED SDA | I2C data — shared bus |
| D3 | GPIO0 | L298N IN1 | Motor Left — Forward direction |
| D4 | GPIO2 | L298N IN2 | Motor Left — Backward direction |
| D5 | GPIO14 | L298N IN3 | Motor Right — Forward direction |
| D6 | GPIO12 | L298N IN4 | Motor Right — Backward direction |
| D7 | GPIO13 | L298N ENA | Left motor PWM speed control |
| D8 | GPIO15 | L298N ENB | Right motor PWM speed control |
| D0 | GPIO16 | IR Sensor Signal | Front obstacle IR — INPUT_PULLUP |
| TX | GPIO1 | Buzzer Signal | Active buzzer — Serial.end() required |
| RX | GPIO3 | FREE | Not used — leave disconnected |
| A0 | ADC0 | NOT USED | Do NOT connect IR here — max 1V only |
| 3.3V | — | MPU6050 + OLED + IR + Buzzer VCC | From Power Board 3.3V rail |
| GND | — | All component GNDs | Common ground rail |
| USB | — | Laptop/PC | Power + programming + dashboard |

⚠ WARNING: Disconnect the TX wire (connected to Buzzer) before uploading code. TX is the serial programming line. Reconnect after upload completes.

✕ DANGER: Do NOT connect the IR sensor signal wire to the A0 (analog) pin. A0 on ESP8266 has a maximum input of 1.0V. A 3.3V IR signal will exceed this limit and may damage the ADC.

## 3.2  MPU-6050 Wiring (Car 2)

| MPU-6050 Pin | Connect To |
| --- | --- |
| VCC | Power Board 3.3V |
| GND | Common GND rail |
| SCL | D1 (GPIO5) |
| SDA | D2 (GPIO4) |
| ADO | GND (sets I2C address 0x68) |
| XDA, XCL, INT | Not connected |

## 3.3  SSD1306 OLED Wiring (Car 2)

| OLED Pin | Connect To |
| --- | --- |
| VCC | Power Board 3.3V |
| GND | Common GND rail |
| SCL | D1 (GPIO5) — shared with MPU-6050 |
| SDA | D2 (GPIO4) — shared with MPU-6050 |

## 3.4  IR Obstacle Sensor Wiring (Car 2)

| IR Sensor Pin | Connect To |
| --- | --- |
| VCC | Power Board 3.3V |
| GND | Common GND rail |
| Signal | D0 (GPIO16) — digital input, INPUT_PULLUP |

ℹ NOTE: IR sensor output: LOW = obstacle detected, HIGH = clear. Code uses INPUT_PULLUP on D0. Mount the sensor at the front center of Car 2 chassis for best obstacle detection coverage.

⚠ WARNING: Do NOT use RX (GPIO3) for the IR sensor. GPIO3 has boot-time activity that causes false trigger readings. D0 is the correct and stable pin for Car 2 IR input.

## 3.5  Active Buzzer Wiring (Car 2)

| Buzzer Pin | Connect To |
| --- | --- |
| VCC / (+) | Power Board 3.3V |
| GND / (−) | Common GND rail |
| Signal | TX (GPIO1) — after Serial.end() in setup() |

ℹ NOTE: TX pin is shared with the Serial port. The firmware calls Serial.end() in setup() to release TX, then reconfigures it as a buzzer output via pinMode(BUZZER_PIN, OUTPUT). Serial is restarted afterward for dashboard output only.

## 3.6  L298N Motor Driver Wiring (Car 2 — 2WD)

| L298N Pin | Connect To |
| --- | --- |
| 12V / VIN | Power Board 12V — motor power |
| 5V / VCC | Power Board 5V — logic power |
| GND | Common GND rail |
| IN1 | D3 (GPIO0) — Left motor forward |
| IN2 | D4 (GPIO2) — Left motor backward |
| IN3 | D5 (GPIO14) — Right motor forward |
| IN4 | D6 (GPIO12) — Right motor backward |
| ENA | D7 (GPIO13) — Left motor PWM speed |
| ENB | D8 (GPIO15) — Right motor PWM speed |
| OUT1/OUT2 | Left motor terminals |
| OUT3/OUT4 | Right motor terminals |

# 4.  ESP-NOW Communication

## 4.1  No Additional Wiring Required

ESP-NOW operates over the ESP8266 built-in 2.4GHz WiFi radio. No additional wiring, modules, or antennas are needed. Communication is entirely software-configured.

## 4.2  MAC Address Configuration

Each ESP8266 has a unique factory-programmed MAC address. Both addresses must be entered in the firmware before uploading. Use the test_espnow sketch to discover each board's MAC address.

| Vehicle | MAC Address | Configured In |
| --- | --- | --- |
| Car 1 — Leader | C4:5B:BE:71:31:47 | car2_bidirectional/src/main.cpp |
| Car 2 — Follower | C8:C9:A3:66:97:D1 | car1_bidirectional/src/main.cpp |

ℹ NOTE: MAC addresses are unique to YOUR boards. The values above are examples from the project build. Always verify using the test_espnow sketch before deploying bidirectional code.

## 4.3  Channel Configuration

Both ESP8266 boards must operate on the same WiFi channel. The firmware explicitly sets channel 1 on Car 2 using wifi_set_channel(1) to match Car 1's default channel.

# 5.  Upload Procedure

## 5.1  Car 1 — Leader Upload Steps

- Ensure Car 1 ESP8266 is connected to laptop via USB cable.

- Disconnect the RX wire from the ESP8266 RX pin (this wire connects to L298N ENB).

- Open car1_bidirectional/ folder in PlatformIO IDE.

- Verify upload_port = COM3 (or your actual COM port) in platformio.ini.

- Click the Upload button. Wait for 'SUCCESS' message.

- Reconnect the RX wire to the ESP8266 RX pin.

- Open Serial Monitor at 115200 baud to verify output.

## 5.2  Car 2 — Follower Upload Steps

- Ensure Car 2 ESP8266 is connected to laptop via USB cable.

- Disconnect the TX wire from the ESP8266 TX pin (this wire connects to Buzzer).

- Open car2_bidirectional/ folder in PlatformIO IDE.

- Verify upload_port = COM3 (or your actual COM port) in platformio.ini.

- Click the Upload button. Wait for 'SUCCESS' message.

- Reconnect the TX wire to the ESP8266 TX pin (Buzzer).

- Open Serial Monitor at 115200 baud to verify /*...*/ output format.

## 5.3  Expected Serial Monitor Output

Car 1 Serial Monitor (115200 baud): [CAR1] DIST: 45cm ZONE:1 TILT:4.6 SPD:160 DIR:0 MSG:2 LINK:OK /*45,1,4.6,160,0,2,0,0,1,2*/ Car 2 Serial Monitor (115200 baud): SCENARIO:5 PLATOON /*1,45,1,160,0,4.6,160,0,0,5*/

# 6.  5 Safety Scenarios Quick Reference

| # | Scenario | Trigger | Car 1 Action | Car 2 Action |
| --- | --- | --- | --- | --- |
|  | Forward Collision | HC-SR04 <= 11cm | Emergency stop + sends EMERGENCY flag | Receives flag → immediate stop + buzzer |
|  | Following Distance | Continuous (zones 0–3) | Sends distance_zone each loop | Maps zone to speed: 220/160/80/0 |
|  | Crash Detection | ¦ax¦ or ¦ay¦ > 12000 | Stops + sends crash_detected=true | Receives flag → stops + beeps long |
|  | Shared Obstacle | Car 2 IR sensor LOW | Receives ir_obstacle → mirrors turn | Steers right + sends ir_obstacle=true |
|  | Platoon Follow | All clear, link active | Full speed forward (PWM 220) | Mirrors Car 1 speed and direction |

ℹ NOTE: Priority order (highest to lowest): Scenario 3 > Scenario 1 > Scenario 4 > Scenario 2 > Scenario 5. Multiple scenarios can be triggered simultaneously — the highest priority takes control.

# 7.  Quick Reference Card

## 7.1  I2C Addresses

| Device | I2C Address |
| --- | --- |
| MPU-6050 | 0x68 (AD0 pin connected to GND) |
| SSD1306 OLED | 0x3C (default for 128×64 module) |

## 7.2  Speed Constants (PWM Values)

| Constant | PWM Value |
| --- | --- |
| SPEED_STOP | 0 |
| SPEED_SLOW | 80 |
| SPEED_MEDIUM | 160 |
| SPEED_FAST | 220 |

## 7.3  Distance Zones (Car 1 HC-SR04)

| Zone | Distance |
| --- | --- |
| Zone 0 — SAFE | Greater than 50cm |
| Zone 1 — WARNING | 20cm to 50cm |
| Zone 2 — DANGER | 11cm to 20cm |
| Zone 3 — CRITICAL | 11cm or less |

## 7.4  Crash Detection Threshold

| Parameter | Value |
| --- | --- |
| Threshold (raw) | 12000 (MPU-6050 raw ADC units) |
| Equivalent (g) | Approximately 0.73g acceleration spike |
| Axes monitored | ax (X-axis) and ay (Y-axis) |
| Formula | ¦ax_raw¦ > 12000 OR ¦ay_raw¦ > 12000 |

## 7.5  Serial Output Format

| Car | Format |
| --- | --- |
| Car 1 | /*DIST,ZONE,TILT,SPD,DIR,MSG,CRASH,BRAKE,LINK,SCENARIO*/ |
| Car 2 | /*LINK,C1DIST,C1ZONE,C1SPD,IR,TILT,C2SPD,CRASH,EMRG,SCENARIO*/ |

V2V Multi-Sensor Fusion Communication System  —  Wiring Guide v1.0  |  Deepak M  |  2025